Specifications

Model: 24PCS*12W 5in1 LED WASH LIGHT LED: 24x 12Watt RGBWA 5in1 Working Position: Any safe working position Voltage: 100V/240V 50/60Hz Power Draw: 300W Casing: Aluminum Alloy Casing Net Weight: 7.5Kg Colors: RGBWA color Mixing DMX Channels: 5/9 DMX channels IP:IP65

24*12W RGBWA

LED WALL WASHER Light

User Instructions

KEYS INSTRUCTION

☞ DIGITAL SHOW FUNCTION FORM

| No | Show | Function | Value | Instruction |
| --- | --- | --- | --- | --- |
| 1 | d001 | DMX address | 001~512 | 5CH mode,UP,DOWN increase or reduce the value, |
| 2 | A001 | DMX address | 001~512 | 9CH mode UP,DOWN increase or reduce the value |
| 3 | r255 | Red dim | 000~255 | UP,DOWN Change brightness |
| 4 | G255 | Green dim | 000~255 | UP,DOWN Change brightness |
| 5 | b255 | Blue dim | 000~255 | UP,DOWN Change brightness |
| 6 | u255 | White dim | 000~255 | UP,DOWN Change brightness |
| 7 | y255 | Amber dim | 000~255 | UP,DOWN Change brightness |
| 8 | FH99 | Strobe | 01 ~ 99 | UP,DOWN change the speed |
| 9 | CL01 | Colors choose | 01 ~ 08 | UP,DOWN Change color |
| 10 | CC99 | Colors Jump | 01 ~ 99 | UP,DOWN change the speed |
| 11 | DE99 | Colors Gradate | 01 ~ 99 | UP,DOWN change the speed |
| 12 | CP99 | Colors Pulse | 01 ~ 99 | UP,DOWN change the speed |
| 13 | SU01 | Colors sound-Active | 01~09 | UP、DOWN Change the effects |

☞ DMX MODE

5Channels  Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| CH1 | R Dimmer | Red Dimmer- Dark to Bright |
| CH2 | G Dimmer | Green Dimmer- Dark to Bright |
| CH3 | B Dimmer | Blue Dimmer- Dark to Bright |
| CH4 | W Dimmer | White Dimmer- Dark to Bright |
| CH5 | A Dimmer | Amber- Dark to Bright |

9 Channels  Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| CH1 | Total Dimmer | RGBW Total Dimmer- Dark to Bright |
| CH2 | R Dimmer | Red Dimmer- Dark to Bright |
| CH3 | G Dimmer | Green Dimmer- Dark to Bright |
| CH4 | B Dimmer | Blue Dimmer- Dark to Bright |
| CH5 | W Dimmer | White Dimmer- Dark to Bright |
| CH6 | A Dimmer | Amber- Dark to Bright |
| CH7 | Total Strobe | RGBW Total Flash Strobe- Slow to Fast |
| CH8 | Function Choice | 0~50: CH1-CH8 Control. 51~100: Different Colors output 101~150: Colors Jump Change. 151~200: Colors Gradate. 201~250: Colors Pulse Change. 251~255: Sound-Active |
| CH9 | Function/Speed | Slow to Fast (At the Colors output and sound choose the color) |